PRESS RELEASE

Wolfurt, on 28.01.2020

Standardised components for strip guiding

Many stamping die components such as strip guiding are usually produced in small quantities, which is quite expensive and time-consuming. That is why using standardised components is a good option. Advantages include immediate availability, the combination possibilities of the different types of strip guiding within the stamping die, and the provided CAD data for the design. The following takes a closer look at the strip guiding solutions available at Meusburger.

E 5632 Spring-loaded strip guiding pin, round

The spring-loaded strip guiding pin with collar can be used for different strip thicknesses and is optimally protected against wear thanks to the hardened design. For dies with small installation space, the spring can be built into the bore in a space-saving manner from a diameter of 10 mm.

Image source: Meusburger, publication free of charge

Caption: Installation example of spring-loaded strip guiding pin, round

E 5634 Spring-loaded strip guide, block-shaped

The block-shaped strip guide is mainly used when there are side cut-outs and therefore too little contact surface available. Due to the intelligent design, it is possible to mount this strip guide directly from the working plane. Thanks to the centrally positioned shoulder screw, only a small amount of installation space is required within the stamping die. The length of the shoulder screw also determines the spring displacement of the strip guide. The symmetrical design allows the block-shaped strip guide to be used on both sides.

Image source: Meusburger, publication free of charge

Caption: Installation example of spring-loaded strip guide, block-shaped

The following installation example shows that the block-shaped strip guide is optimally suited for side cut-outs. In addition, a further advantage of using standardised components is demonstrated: the different strip guides are optimally matched to each other and can therefore be combined as required within the stamping dies.

Image source: Meusburger, publication free of charge

Caption: Optimal for strips with side cut-outs

Image source: Meusburger, publication free of charge

Caption: Spring-loaded strip guide, block-shaped

E 5636 Strip guiding rail

The strip guiding rail is always used in combination with a strip lifter. Thanks to the screw connection on the top, it can be directly installed from the working plane. The hardened design ensures the best wear protection and the integrated infeed chamfers allow safe insertion of the punching strip. In addition, it can be used on both sides thanks to the symmetrical design.

Image source: Meusburger, publication free of charge

Caption:  E 5636 Strip guiding rail with E 5640 Strip lifter, round

E 5640 Strip lifter, round

The small diameter versions are suitable for dies with limited installation space. The integrated initial bore enables the introduction of a clearance hole for a pilot pin. In addition, the lengths of the strip lifter with collar are matched to the strip guiding pin with collar - which makes it easy to combine these two elements.

E 5644 Strip lifter, block-shaped

The large contact surface of the block-shaped strip lifter guarantees reliable lifting and forward pushing of the metal strip, even for large cut-outs. The E 5644 also has an integrated initial bore for the introduction of a clearance hole for pilot pin, and all functional dimensions are matched to the respective plate thicknesses. All items are available from stock. For more information, please visit www.meusburger.com.

Image source: Meusburger, publication free of charge

Caption: Block-shaped strip lifter

Meusburger – Setting Standards.

Meusburger is the market leader in the field of high-precision standard parts. Customers all over the world make use of the advantages of standardisation and benefit from the company’s over 50 years of experience in working with steel. The product portfolio ranges from high-precision standard parts and selected products in the field of workshop equipment to hot runner and control systems, the WBI Knowledge Management Method and solutions for efficient corporate management in the ERP/PPS software area. This makes Meusburger the reliable global partner for making moulds, dies, jigs and fixtures.

Press contact

Meusburger Georg GmbH & Co KG

Communication / Public relations

Phone: +43 5574 6706-0

Email: press@meusburger.com

www.meusburger.com